USNG

10xxxxxx.1

# LEGAL RESTRICTIONS:

This Reusable Software Component (RSC) contains data with Unlimited Government Rights.

# DESCRIPTION:

USNG is a C language code component that provides conversions between Geodetic coordinates (latitude and longitude), Universal Transverse Mercator (UTM) coordinates, Universal Polar Stereographic (UPS) coordinates, and United States National Grid (USNG) coordinates.

A particular ellipsoid is specified in terms of the following parameters:

- Semi-Major Axis (a):	Radius (in meters) at the equator,

- Flattening (f):	Ratio of the difference between the semi-major axis and polar radius of the Earth to its semi-major axis.

- Ellipsoid Code		Standard 2-letter ellipsoid code.

# CERTIFICATION LEVEL:

- This RSC has been certified at level 4.  A level 4 component satisfies the criteria for reliability, testing, and documentation for the Army Reuse Center (ARC).  The component comes with test materials and a Reuse Manual that aids in integrating the component into a software system.

# LEVEL OF TESTING/ACCEPTANCE:

- Unit and integration testing have been performed for the functions contained in this component.

# PURPOSE/INTENDED USE:

- The purpose of USNG is to provide a reusable component which supports the following coordinate conversions:

- Geodetic coordinates (latitude and longitude in radians) to United States National Grid (USNG) coordinate string, and

- United States National Grid (USNG) coordinate string to Geodetic coordinates (latitude and longitude in radians).

- Universal Transverse Mercator (UTM) coordinates (zone, hemisphere, easting and northing in meters) to USNG coordinate string.

- USNG coordinate string to Universal Transverse Mercator (UTM) coordinates (zone, hemisphere, easting and northing in meters).

- Universal Polar Stereographic (UPS) coordinates (hemisphere, easting and northing in meters) to USNG coordinate string.

- USNG coordinate string to Universal Polar Stereographic (UPS) coordinates (hemisphere, easting and northing in meters).

# HARDWARE/ENVIRONMENT CONSTRAINTS:

There are no hardware or environment constraints.  There are no limitations.

# FUNCTIONS:

Set_USNG_Parameters – This function sets the ellipsoid parameters to be used in subsequent coordinate conversion operations.

Get_USNG_Parameters – This function returns the current values of the ellipsoid parameters.

Convert_Geodetic_To_USNG – This function converts the specified geodetic coordinates (latitude and longitude in radians) to the corresponding USNG coordinate string with the specified level of precision.

Convert_USNG_To_Geodetic – This function converts the specified USNG coordinate string to geodetic coordinates (latitude and longitude in radians).

Convert_UTM_To_USNG – This function converts the specified UTM coordinates (zone, hemisphere, easting and northing in meters) to the corresponding USNG coordinate string with the specified level of precision.

Convert_USNG_To_UTM – This function converts the specified USNG coordinate string to UTM coordinates (zone, hemisphere, easting and northing in meters).

Convert_UPS_To_USNG – This function converts the specified UPS coordinates (hemisphere, easting and northing in meters) to the corresponding USNG coordinate string with the specified level of precision.

Convert_USNG_To_UPS – This function converts the specified USNG coordinate string to UPS coordinates (hemisphere, easting and northing in meters).

# EXAMPLE ApplicationS:

The following example illustrates how USNG can be used to convert Geodetic coordinates to USNG coordinates and back again:

## Function Call:

status = Set_USNG_Parameters (a, f, Ellipsoid_Code)

## Inputs:

- a	6378137.0

- f	1 / 298.25722210100

- Ellipsoid_Code	“RF”

## Function Call:

status = Convert_Geodetic_To_USNG (Latitude, Longitude, Precision, USNG)

## Inputs:

- Latitude:	35.0

- Longitude:	-75.0

- Precision	5

## Outputs:

- USNG:	18SWD0000073043

## Function Call:

status = Convert_USNG_To_Geodetic (USNG, Latitude, Longitude)

## Inputs:

- USNG:	18SWD0000073043

## Outputs:

- Latitude:	35.0

- Longitude:	-75.0