GEOCENTRIC

10xxxxxx.1

# LEGAL RESTRICTIONS:

This Reusable Software Component (RSC) contains data with Unlimited Government Rights.

# DESCRIPTION:

GEOCENTRIC is a C language code component that provides conversions between Geodetic coordinates (latitude, longitude in radians and height in meters) and Geocentric coordinates (X, Y, Z in meters).

# CERTIFICATION LEVEL:

This RSC has been certified at level 4.  A level 4 component satisfies the criteria for reliability, testing, and documentation for the Army Reuse Center (ARC).  The component comes with test materials and a Reuse Manual that aids in integrating the component into a software system.

# LEVEL OF TESTING/ACCEPTANCE:

Unit and integration testing have been performed for the functions contained in this component.

# PURPOSE/INTENDED USE:

The purpose of GEOCENTRIC is to provide a reusable component which supports the following coordinate conversions:

- Geodetic coordinates (latitude, longitude in radians and height in meters) to Geocentric coordinates (X, Y, Z in meters),

- Geocentric coordinates (X, Y, Z in meters) to Geodetic coordinates (latitude, longitude in radians and height in meters).

A particular ellipsoid is specified in terms of the following parameters:

- Semi-Major Axis (a) – Radius (in meters) at the equator.

- Flattening (f):	Ratio of the difference between the semi-major axis and polar radius of the Earth to its semi-major axis.

# HARDWARE/ENVIRONMENT CONSTRAINTS:

There are no hardware or environment constraints.  There are no limitations.

# FUNCTIONS:

Set_Geocentric_Parameters – This function sets the ellipsoid parameters that are to be used in subsequent coordinate conversion operations.

Get_Geocentric_Parameters – This function returns the current values of the ellipsoid parameters.

Convert_Geodetic_To_Geocentric – This function converts the specified Geodetic coordinates (latitude, longitude in radians and height in meters) to Geocentric coordinates (X, Y, Z in meters) using the current ellipsoid parameters.

Convert_Geocentric_To_Geodetic – This function converts the specified Geocentric coordinates (X, Y, Z in meters) to Geodetic coordinates (latitude, longitude in radians and height in meters) using the current ellipsoid parameters.

# EXAMPLE ApplicationS:

The following example illustrates how GEOCENTRIC can be used to convert Geodetic coordinates to Geocentric coordinates and back again:

Function Call:

status = Set_Geocentric_Parameters (a, f)

## Inputs:

- a	6378137.0

- f	1 / 298.257223563

## Outputs:

- none

## Function Call:

status = Convert_Geodetic_To_Geocentric (Latitude, Longitude, Height, X, Y, Z)

## Inputs:

- Latitude:	45.0

- Longitude:	-75.0

- Height:	700

## Outputs:

- X:	1169366.666323

- Y:	-4364135.811395

- Z:	4487843.383565

## Function Call:

status = Convert_Geocentric_To_Geodetic (X, Y, Z, Latitude, Longitude, Height)

## Inputs:

- X:	1169366.666323

- Y:	-4364135.811395

- Z:	4487843.383565

## Outputs:

- Latitude:	45.0

- Longitude:	-75.0

- Height:	700.004645